# Automation of Concrete Mix Design

# Nebraska Department of Transportation

# CIVE 202-150 – Civil Engineering Analysis II

Carter Janssen, Luke Moore, Adam Meismer

February 24, 2026

Introduction

The Nebraska Department of Transportation currently utilizes an Excel based workflow to perform concrete mix design calculations. While the spreadsheet provides accurate results, it is manually operated and dependent on user interpretation of embedded formulas. NDOT has requested that the logic contained within the Mix Design Excel worksheet be translated into a Python based system that replicates all calculations while improving transparency, repeatability, and reliability. Therefore, our group translated the Excel formulas into Python functions that prompt the user for inputs sequentially and generate a weight chart for one cubic yard of concrete. In addition, four realistic concrete mix scenarios were researched and evaluated using the automated workflow to verify proper implementation and compare performance characteristics.

Methods

Our team started the automation process with a detailed review of the Mix Design tab of the NDOT Excel workbook to identify all input parameters, intermediate calculations, and final outputs. Each formula was documented and categorized into logical calculation blocks, including water content calculation, cementitious material volumes, air and water volumes, total aggregate volume, and aggregate weight computations. This mapping ensured that every spreadsheet dependency was captured prior to coding. The translation from Excel to Python was done using a modular approach to program. Instead of using one long script, each major calculation block was made into a dedicated Python function. Water weight was calculated as a function of total cementitious materials and the specified water–cement ratio. Cementitious volumes were computed by dividing material weights by the product of specific gravity and unit weight of water (62.4 lb/ft³).  The air volume was calculated by multiplying 27 ft³ to the Air% divided by 100. The Water volume was found by dividing the water weight by 100. The total aggregate volume was determined by subtracting sum of cementitious volumes, air volume, and water volume from 27 ft³ per cubic yard. Aggregate weights were then calculated using aggregate volume, percentage proportions, and respective specific gravities. All these calculations match the excel calculations exactly.  Sequential user input prompts were implemented using structured input() statements to mirror the layout of the Excel worksheet (Figure 1).

| Input Parameters | Output Parameters |
| --- | --- |
| Cement weight (lb/yd³) | Water weight (lb) |
| Fly ash weight (lb/yd³) | Cementitious material volumes (ft³) |
| Silica fume weight (lb/yd³) | Air volume (ft³) |
| Other supplementary cementitious material (lb/yd³) | Water volume (ft³) |
| Water–cement ratio | Total aggregate volume (ft³) |
| Air content (%) | Fine aggregate weight (lb) |
| Fine aggregate percentage (%) | Coarse aggregate weight (lb) |
| Coarse aggregate percentage (%) | Other aggregate weight (lb) |
| Other aggregate percentage (%) |  |
| Specific gravities for each cementitious material |  |
| Specific gravities for each aggregate |  |

Figure 1: Input and Output Parameters in Sequential Order

Verification was performed by running identical input sets through both the original Excel worksheet and the Python script. Intermediate results and final weight charts were compared to ensure consistency within rounding tolerances and both showed the same results. After validation, four realistic mix scenarios were evaluated using the automated model to demonstrate its applicability to different engineering contexts.

Results & Discussion

Four concrete mix designs were evaluated using the automated Python workflow (SCN-1 through SCN-4). Each scenario was defined using the same set of required model inputs (Figure 2). The model then produced a final 1-cubic-yard weight summary for cementitious materials, water, and three aggregate bins (fine, coarse, other), which is the required output format for the NDOT deliverable (Figure 3).

| Scenario | Class | Cement | Fly Ash | Silica Fume | Other SCM | w/c ratio | Air % | Fine % | Coarse % | Other % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCN-1 | 47B | 600 | 0 | 0 | 0 | 5.0 | 5.0 | 45 | 55 | 0 |
| SCN-2 | 47BR | 560 | 40 | 0 | 0 | 6.0 | 6.0 | 43 | 57 | 0 |
| SCN-3 | 47B | 520 | 60 | 20 | 0 | 5.5 | 5.5 | 40 | 55 | 5 |
| SCN-4 | 47BR | 580 | 0 | 0 | 20 | 0.47 | 4.5 | 47 | 50 | 3 |

Figure 2: Model Inputs Used in Python

| Scenario | Class | Cement | Fly Ash | Silica Fume | Other SCM | Water | Fine Agg | Coarse Agg | Other Agg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCN-1 | 47B | 600 | 0 | 0 | 0 | 4 | 17 | 17 | 17 |
| SCN-2 | 47BR | 560 | 40 | 0 | 0 | 4 | 17 | 17 | 17 |
| SCN-3 | 47B | 520 | 60 | 20 | 0 | 4 | 17 | 17 | 17 |
| SCN-4 | 47BR | 580 | 0 | 0 | 20 | 4 | 17 | 17 | 17 |

Figure 3: Final Output Weight Summary

SCN-1 represents a traditional NDOT Class 47B concrete mix relying entirely on Portland cement as the binder. The moderate water–cement ratio provides a balance between strength and workability, while 5% air content provides freeze-thaw durability suitable for Nebraska climates. This mix serves as a baseline for comparing sustainability, strength potential, and durability improvements in the other scenarios.

SCN-2 partially replaces Portland cement with fly ash while slightly reducing the water–cement ratio. The addition of 40 lb/yd³ of fly ash lowers the required cement content and improves long-term durability. The lower w/c indicates a higher potential compressive strength, and the slightly higher air content enhances freeze-thaw resistance, making this mix well suited for pavement applications.

SCN-3 introduces a more advanced SCM system by incorporating both fly ash and silica fume. This combination significantly alters the paste microstructure. The w/c ratio of 0.40 is the lowest among all four scenarios, indicating the highest compressive strength potential. This mix would be appropriate for high-performance structural elements, bridge decks, or environments with aggressive exposure conditions.

SCN-4 increases the water–cement ratio to 0.47, which generally reduces compressive strength but improves workability. The lower air content slightly reduces freeze-thaw resistance relative to other scenarios. This mix may be suitable for applications where workability and ease of placement are prioritized over maximum structural performance.

Overall, SCN-3 provides the highest technical performance potential, SCN-2 offers the best balance of sustainability and durability, SCN-1 serves as a reliable baseline, and SCN-4 prioritizes workability over strength optimization.

From the Final output weight summary (figure 3) across the four scenarios, the cementitious breakdown changes as expected based on the inputs. This confirms the program is correctly passing scenario inputs through to the final reporting section and producing consistent formatted summaries for each design. The water and aggregate weights are identical across all four scenarios with water being 4lb and all aggregates being 17lbs each.

References

Barrett. NDOT Pavement Design Manual. 2018, pp. 2–9, dot.nebraska.gov/media/jjwpyezr/pavdesignmanual.pdf.

Oie, Claude. “ADDENDUM NO. 3 AND ELECTRONIC BIDDING SYSTEM AMENDMENT NO. 1.” STATE OF NEBRASKA DEPARTMENT OF ROADS, 12 Mar. 2002, www.nebraskatransportation.org/letting/c-let-020314/f07ad303.pdf.

Materials - NDOT. dot.nebraska.gov/business-center/materials.

Janssen, C, Moore, L, Meismer, A. (2026). CIVE202 Spring 2026 Project 2 Python Code Notebook. Submitted as part of course deliverables.

NDOT Mix Design.xlsx